12. Building services engineering (BSE) systems

# Worksheet 1: Air conditioning components (tutor)

Complete the table below by matching the description to the AC component listed:

cooling coil, recirculation duct, preheater battery, supply fan, reheater battery

| Preheater battery | In the winter cycle fresh air is heated to 7OC preventing the filters from freezing. |
| --- | --- |
| Supply fan | The duty of this fan is slightly greater than that of the return air fan which gives a positive air pressure in the room. |
| Recirculation duct | Up to 25% of room air can be mixed with the fresh air, utilising already conditioned air. |
| Reheater battery | The air is heated to the supply air temperature (34ºC) for comfort and to compensate for heat loss throughout the building. |
| Cooling coil | Cooled by either chilled water or refrigerant gas. Moisture is removed from the air by dehumidification. |

Use this list to label the diagram below.

preheater battery, supply fan, cooling coil, fresh air inlet, reheater battery, exhaust air outlet, recirculation duct, supply air, return air, return air fan