12. Building services engineering (BSE) systems

# Worksheet 7:Cold water systems (tutor)

- Sketch a direct cold water system.

- Sketch an indirect cold water system.

- What is the difference between a direct and an indirect system?

In a direct system of cold water supply, all the sanitary appliances and the cold water storage cistern are connected to the rising main and are therefore supplied with fresh water at mains pressure.

In an indirect system, the only tap connected to the main supply is at the kitchen sink for food preparation and drinking water.