2. Construction science principles

# Worksheet 17: Approved document A – Structure (tutor)

The building regulations approved documents control the standards of construction work.

Download Part A of the regulations from the following link:

https://www.planningportal.co.uk/info/200135/approved_documents/62/part_a_-_structure

Answer the following questions.

1. What determines the depth of a foundation?

Page 36 – its projection from the face of a wall

2. What is the minimum depth of a foundation?

Page 37 – 0.45m

3. How many joists should a galvanised strap span for lateral restraint?

Page 30 – 3

4. What is the width of a 60KN/m loaded foundation on sand and gravel?

Table 10 –  600mm wide

The tutor may wish to give learners the page numbers to locate the information to speed things up.