# 5. Sustainability principles

# Worksheet 19: Energy production and use (tutor)

- Explain what biomass is in relation to energy use.

Biomass is a renewable energy fuel that Is widely used and available in the UK. Biomass is biological material derived from living or recently living organisms. Biomass for energy predominantly means plant-based material but biomass can refer to materials derived from both animals and vegetables.

- Describe the main difference between biomass and fossil fuels.

The key difference between biomass and fossil fuels is timescale. Biomass is harvested as part of a constantly replenished crop if managed on a sustainable basis.

- Explain what is meant by the term carbon capture and storage.

Carbon capture and storage (CSS) is a way of reducing carbon emissions. CSS involves the capture of carbon dioxide emissions from industrial processes.

- Describe the three-step process to carbon capture and storage.

Capture – The carbon dioxide is separated from the other gases produced in the industrial process.

Transport – The carbon dioxide is then compressed and transported via pipelines, ships or road transport to a storage site.

Storage – The carbon dioxide is finally injected into rock formations underground for permanent storage.

- State three disadvantages of burning fossil fuels.

Answers may include:

- Dangerous to produce

- Non-renewable

- Smog and air pollution

- Global warming

- Acid rain

- Sketch a diagram to represent how a solar PV system works.

Learners may sketch the diagram from the presentation below:

- State the name that is often used when referring to biomethane.

Biomethane is often referred to as green gas.

- Provide a definition for the term low carbon power.

Low-carbon power is electricity produced with substantially lower greenhouse gas emissions than conventional fossil fuel power generation. The energy transition to low-carbon power is one of the most important actions required to limit climate change.

- Describe the three types of rainwater systems.

- Gravity – Collected rainwater is pumped from the main holding tank to an elevated/header tank. The connected appliances are then supplied from the tank as in a normal gravity-fed system.

- Direct (pressurised) – Collected rainwater is pump-fed directly from the main holding tank to the serviced appliances.

- Combination – Collected rainwater is pumped or gravity fed to a low-level break tank. In turn, the water is then gravity fed to an integral or external booster pump system for onward distribution to the serviced appliances.

- State three advantages of a rainwater harvesting system.

Answers may include:

- It saves money by reducing water usage.

- Rainwater is better for your garden as it has a balanced pH and is free of chemicals such as chlorine.

- It gains eco home rating for a property.

- It can produce drinking water for livestock and for irrigating crops.