8. Construction information and data principles

# Worksheet 13: Drawings, circuit diagrams and schematics (tutor)

- Which paper size is the largest: A6, A3, A0?

- A0

- How would you know your were looking at an amended drawing?

The drawing number would have an REV number/letter.

- What information would you expect to see on a site plan?

- Notes highlighting changes from previous revisions

- North arrow

- Key dimensions

- Materials

- Site boundaries and delineation of adjacent properties, location of the building or buildings in relation to their surroundings

- Trees, tree protection orders, main elements of the landscape

- Parking areas with dimensions or capacities, traffic flows and signage.

- Roads, footpaths, ramps, paved areas.

- Easements

- What’s the difference between a schematic and a wiring diagram?

- A schematic shows the plan and function for an electrical circuit, but is not concerned with the physical layout of the wires.

- A wiring diagram is a simple visual representation of the physical connections and physical layout of an electrical system or circuit.

- What is the BS to which all electrical installations should conform?

BS7671