5.Sustainability principles

# Worksheet 8: BREEAM certification case study (tutor)

Read the case study about 21 Moorfields, London on the BREEAM website here:

https://www.breeam.com/case-studies/21-moorfields-london/

.

The BREEAM-certified project is a mixed-use development above Moorgate Station, the new Crossrail station and its entrance. The two buildings will provide office space, retail space, a replacement city walkway between Moorfields and the Barbican, a new public square, cycle parking, servicing, storage, plant, landscaping and associated works.

- What was the team’s aspiration for the project?

The aspiration for the development was to create a building that optimises energy efficiency, uses low embodied carbon, and has excellent health and wellbeing credentials.

- For each of the following categories, list one sustainable feature of the development:

  - Energy efficiency

Learners could have listed:

- High efficiency LED lighting

- Occupancy and daylight controls to significantly reduce the lighting emissions

- HVAC plant efficiencies; high efficiency gas-fired boilers & chillers

- Variable speed pumps and drives; demand driven controls

  - Land use and ecology

Learners could have listed:

- Nesting or roosting opportunities for birds or bats within the building

- Trees of significant ecological value protected (as defined by BS 5837:2012 and confirmed by the Suitably Qualified Ecologist or qualified arboriculturalist)

- Planted sedum/evergreen hedges/silver birch trees to be used on the roofs

- Planting palette, designed for planting communities and attracting wildlife

- Butterfly, wild meadow planting

- Insect hotels, mixed planting

- Ground nesting bird zone, aggregate, sedum, shrubbery planting

- Name two kinds of sustainable materials used in the development.

Learners could have listed:

- Low carbon & high recycled content (RC) materials to be used without compromising the quality.

- Steel: High RC (EAF steel) will be used where possible as it is both a lower carbon and more resource efficient practice.

- Concrete: High rates of cement replacement with GGBS used in piling and concrete infill of the composite metal decking floor slabs.

- Lightweight aggregate: Lightweight recycled aggregate will be used for concrete on metal decking floor slabs to reduce the weight.

- Aluminium: High RC and manufactured using renewable energy, anodised finish avoided where possible.

- Insulation: Low impact natural materials preferred e.g. mineral wool.

- Finishes: Low carbon & high RC materials to be used: partitioning boards, floor & ceiling finishes, paint, etc.

- Landscape: Natural & locally sourced stones preferred; granite to be avoided, where possible, due to high carbon intensity.

- Why did the project aim to use local heavyweight materials where possible?

Local sourcing for heavyweight materials reduces transport-related emissions.

- Describe how the building was designed for reduced water consumption.

- The building will use water efficient taps, shower heads, dual flush toilets and low water use appliances.

- Native or adapted plant species will be planted, with high efficiency irrigation systems (drip and sprinkler irrigation and soil moisture sensors) to reduce the use of water.

- Water metering and leak detection alarms will be installed to monitor and minimise water waste.