7. Building technology principles

# Worksheet 16: Approved documents (tutor)

| Part title | Description |
| --- | --- |
| Part A Structure | Requires all buildings to be designed, constructed or altered to be structurally safe and not impair the structural stability of surrounding buildings. |
| Part B Fire safety | Covers all measures to provide safety from fires for building residents, people in the vicinity of the building and firefighters. Guidance covers means of escape, fire detection and warning systems. |
| Part C Site preparation and resistance to contaminants and moisture | Covers weather and water tightness of buildings. Subsoil drainage, site preparation and contaminated land as well as all other related hazardous and dangerous substances. |
| Part D Toxic substances | This controls hazards from the toxic chemicals used in cavity fill insulation systems. |
| Part E Resistance to the passage of sound | Requirements for sound insulation between buildings including new dwellings and conversion of existing buildings to form dwellings. Sound reduction between rooms and acoustic conditions for common areas in flats and schools. |
| Part F Ventilation | Covers standards for ventilation and air quality for all buildings. Condensation is also covered. |
| Part G Sanitation, hot water safety and water efficiency | Sets out standards for provision of sanitary and washing facilities. It also covers bathrooms and hot water systems and safety requirements for unvented hot water systems. |
| Part H Drainage and waste disposal | Provides guidance on adequate drainage and pollution prevention. Technical design standards cover sanitary pipework, foul drainage, rainwater drainage and disposal, wastewater treatment, discharges and cesspools. |
| Part J Combustion appliances and fuel storage systems | Covers the construction and installation of boilers, chimneys, flues, hearths and fuel storage installation. Also covers requirements to control fire sources and prevent burning, pollution and carbon monoxide poisoning. |
| Part K Protection from falling, collision and impact | Sets standards for safety of stairways, ramps and ladders. Includes guarding against and warning of hazards from use and position of doors and windows. |
| Part L Conservation of fuel and power | Controls the insulation values of building elements. Covers mechanical ventilation and air conditioning systems and carbon index ratings. |
| Part M Access to use of buildings | Covers the inclusive provision of ease of access to and circulation in all buildings along with requirements for facilities for disabled people. |
| Part P Electrical safety | Covers the design, installation, inspection and testing of electrical installations in order to prevent injuries from electrical shocks, burns and fires due to electrical components overheating. |
| Part Q Security | Provision must be made to resist unauthorised access to any dwelling and any part of a building from which access can be gained to a dwelling in the building. |
| Part R Physical infrastructure for high speed electronic communications networks | A new requirement for physical infrastructure which enables connections to broadband networks. |

Can you also recall what is covered by Regulation 7?

Covers the use of appropriate materials for a construction and how people working on the building must behave.