7. Building technology principles

# Worksheet 17: Building regulations (tutor)

- Explain the purpose of building regulations.

Set minimum standards and expectations for both the design and construction of buildings. Ensure the safety of the building as well as the health and safety of the people who occupy or use the building.

- Explain how building regulations are enforced

By the auditing and checking of construction documentation including plans and specifications that are submitted to the local authority. Building control officers can also enforce during inspections of building works onsite.

- List the roles and responsibilities of a building control officer.

Checking of plans for approval

Giving advice on construction

Maintaining an approved standard

Inspections

Dealing with dangerous structures

Demolition notices

Enforcement work

- What legislation is associated with building regulations?

Building Act 1984, Sustainable and Secure Buildings Act 2004.

- Explain the possible consequences of failing to abide by building regulations.

Can result in prosecution and unlimited fines. The local authority has the power to order non-compliant building work to be removed or rectified.

- Give two advantages of building notices.

Plans do not need to be drawn up by a designer.

Purchase and preparation of a site plan is all that is required initially.

- Explain the possible consequences of unauthorised building work .

It is an offence to carry out notifiable building work without submitting full plans or a building notice application. This can lead to prosecution and will almost certainly create difficulties for the sale of a property.

- Sketch a diagram to demonstrate at what points inspections are carried out.

Flow chart showing inspection intervals.