3. Engineering representations

# Worksheet 10: More engineering symbols (tutor)

Complete the worksheet by drawing the symbol for each engineering term, and give its purpose/indication.

| Name | Symbol | Purpose/indication |
| --- | --- | --- |
| Slope |  | Placed at the left end of the slope specification |
| Straightness | _____ | Indicates a straightness requirement |
| Flatness |  | An oblique view of a square surface |
| Circularity |  | Indicated by a circle |
| Cylindricity |  | Two parallel lines drawn tangent to the circle |
| Parallelism |  | Two parallel straight lines |
| Perpendicularity |  | Two lines at right angles |
| Angularity |  | Two lines forming a 30 degree angle |
| Concentricity |  | Two concentric circles |
| Surface finish |  | Level of surface finish |
| PCD |  | The diameter of the circle on which holes will be drilled in a circular pattern |
| Datum |  | The point/side/face from which all measurements should be taken |
| Hexagon head | HEX | Shows a hexagonal bolt |
| Undercut | UCUT | Shows an undercut |
| Thread | THD | Indicates a thread |