3. Engineering representations (tutor)

Using the table provided on the next page, calculate the upper and lower limit of both the shaft and the bore. Can you infer the type of fit from your calculations?

|  | Bore size | Shaft size |
| --- | --- | --- |
| Type of fit = Clearance | + 0.062 + 0.000 = 42.062 42.000 | – 0.080 – 0.180 = 41.980 41.820 |
| Type of fit = Transition | + 0.030 + 0.000 = 57.030 57.000 | + 0.039 + 0.020 = 57.039 57.020 |
| Type of fit = Interference | + 0.035 + 0.00 = 80.035 80.00 | + 0.078 + 0.059 = 80.078 80.059 |