3. Engineering representations

# Worksheet 8: Symbols and abbreviations (tutor)

- Complete the table below, filling in either the abbreviation, symbol or the terms that are missing.

|  | Term | Abbreviation |
| --- | --- | --- |
| 1 | Across flats | AF |
| 2 | Centres | CRS |
| 3 | Countersink | CSK |
| 4 | Chamfer | CHAM |
| 5 | Counterbore | CBORE |
| 6 | Revision | Rev |
| 7 | External | EXT |
| 8 | Internal | INT |
| 9 | Not to scale | NTS |
| 10 | Spot face | SFACE |
| 11 | Specification | SPEC |
| 12 | Thickness | THK |
| 13 | Standard | STD |
| 14 | Pitch circle diameter | PCD |
| 15 | Centre line | CL |
| 16 | 1st angle projection |  |
| 17 | 3rd angle projection |  |

- Complete the table below to identify different types of line. Sketch the line type and describe it, then state how it is used.

| Line type | Description of line type/drawing | How line is used |
| --- | --- | --- |
| Drawing line | Continuous | Used to show visible outline and edges |
| Hidden line | Dashed | To show detail hidden by another feature |
| Centre line | Long line, gap, short line gap, long line | Shows the centre of a hole or cylinder-shaped object |
| Centre mark | A simple cross | Marks the centre of a radius, hole or circle |
| Phantom line | Dash and dotted line | Used to indicate imaginary features not depicted, eg alternate positions of moving parts |
| Break line | Continuous line with breaks shown as jagged movement. | To show an interrupted line |
| Cutting plane | Dashed line with perpendicular arrows at each end. | Used to indicate where the section or cut is made to see interior detail |
| Dimension line | Solid line with arrows at either end | Shows dimension between two points |
| Leader line | Arrow line or line running straight with gap of 1- mm from feature | Information or measurements pointing to an object |