4. Essential mathematics for engineering and manufacturing

# Worksheet 11.1: Arithmetic progressions (tutor)

| First term (a) | Common difference (d) | First 5 terms of the sequence |
| --- | --- | --- |
| 4 | 6 | 4, 10, 16, 22, 28 |
| 10 | −9 | 10, 1, −8, −17, −26 |
| −2 | 8 | −2, 6, 14, 22, 30 |
| −1 | 4 | −1, 3, 7, 11, 15 |
| 0 | 10 | 0, 10, 20, 30, 40 |
| −8.3 | 0.3 | −8.3, −8, −7.7, −7.4, −7.1 |
| 5 | 1 | 5, 6, 7, 8, 9 |
| −7 | 6 | −7, −1, 5, 11, 17 |

Contextual questions

- A milling drill is used to cut out parts of an engineering component. The starting temperature of the drill is 18 °C and, when in use, it heats up at a rate of 1.2 °C a minute. a.	Determine the temperature of the drill after :

- 5 minutes

- 18 + (5 × 1.2) = 24°C

- ii.	7 minutes 30 seconds

- 18 + (7.5 × 1.2) =27°C

- iii.	9 minutes 50 seconds.

- 18+956×1.2=29.8℃

- b.	When the temperature reaches 62 °C, the drill will stop for safety reasons. Calculate 
the maximum length of time the drill can be used for. Give your answer to the nearest second.

- 62=18+(t×1.2)

- t=62-181.2=36.67 minutes.

Plastic guards are moulded and stacked ready for distribution. One mould has a height of 56 mm and each additional mould adds 12 mm to the height.

- Determine the height of 20 stacked moulds.

- 56+19×12=284 mm

- Determine the height of 15 stacked moulds

- 56+14×12=224 mm

- The boxes used for packing have a height of 60 cm. Calculate the maximum number of moulds that can be stacked.

- 600=56+n-112

- 600=56+12n-12

- 600=12n+44

- n=600-4412=46.3333

- The maximum number of guards is 46.

A 3D printer takes 40 seconds to warm up ready for printing and then 90 seconds to print a rawl plug.

- By generating an appropriate sequence, show that the time taken (t) to print n plugs is given by t=90n+40.

| - n | - 1 | - 2 | - 3 | - 4 |
| --- | --- | --- | --- | --- |
| - t | - 130 | - 220 | - 310 | - 400 |

- a = 130,  d = 90

- t = 130 + (n-1)90

- t=130+90n-90

- t=90n+40

Oil is added to a machine at a rate that is proportional to the mass of aggregate that is being moved. The initial amount of oil present in the machine is 2 litres and extra oil at a rate of 1.6 litres per tonne of aggregate is then required.

- By generating an appropriate sequence, show the amount of oil required for all masses (in 250 kg increments) from 0 to 4 tonnes.

- a=2 , d=0.4l per 250 kg or 1.6 l per 1000 kg (tonne) or 0.0016 l per kg

- Let o be the amount of oil needed and let m be the mass of the aggregate.

- o=2+0.0016m

| m (kg) | o (litres ) |
| --- | --- |
| 250 | 2.4 |
| 500 | 2.8 |
| 750 | 3.2 |
| 1000 | 3.6 |
| 1250 | 4.0 |
| 1500 | 4.4 |
| 1750 | 4.8 |
| 2000 | 5.2 |
| 1250 | 5.6 |
| 2500 | 6.0 |
| 2750 | 6.4 |
| 3000 | 6.8 |
| 3250 | 7.2 |
| 3500 | 7.6 |
| 3750 | 8.0 |
| 4000 | 8.4 |

A crane is hired to complete a building project. The cost of hire is £300 for the first day and then an additional £75 each day thereafter. The crane can lift and place 1 beam every 20 minutes. A working day consists of 8 hours.

- By generating an appropriate arithmetic sequence and then applying the formula for the nth term, find formulae linking the following variables.

- Cost and number of days hire.

- a=300,  d=75

- Let C be the cost and d the number of days hire.

- C=300+d-175

- C=300+75d-75

- C=75d+225

- Number of beams needing to be placed and time taken.

- Let d be the number of days.

- Let b be the number of beams.

- d=b24

- Number of beams needing to be placed and the cost of the crane hire to move them.

- C=(75×b24)+225