4. Essential mathematics for engineering and manufacturing

# Worksheet 18: Circular measure with radians (tutor)

- Work out the area and perimeter of each of the following sectors. Give your answers correct to 3 significant figures.

- A=12×142×7π12=180 m2		Perimeter=14+14+(14×7π12)=53.7 m

- A=12×1.92×5π6=4.73 mm2		Perimeter=1.9+1.9+1.9×5π6=8.77 mm

- A=12×0.62×0.7=0.126 m2		Perimeter=0.6+0.6+0.6×0.7=1.62 m

- A=12×42×2.6=20.8 cm2		Perimeter=4+4+4×2.6=18.4 cm

- Change the degrees to radians→230×2π360=23π18 rad

A =12×652×23π18=8480 mm2	Perimeter=65+65+65×23π18=391 mm

- Change the degrees to radians →90×2π360=π2 rad

- A=12×2.872×π2=6.47 cm2		Perimeter=2.87+2.87+2.87×π2=10.2 cm

Contextual questions

- Below is a plate made of steel.

- Find the area of the plate to the nearest mm2.

- 100°=5π9 rad

- Asector=12×952×5π9=45125π18 mm2

- Acircle=π×52=25π

- Area of plate= 45125π18-25π-25π=44225π18 mm2           (7718.718…mm2)

- Area = 7719 mm2 to the nearest mm2

- The plate is made by cutting out the sector first and then stamping out the two circular sections. What percentage of the metal is wasted?

- % waste= 50π45125π18×100=1.99%

- The plate is welded around the edge of the sector. 
What length of welding needed for one plate?

- Perimeter=95+95+95×5π9=356 mm

A 20 cm length of wire is bent into the shape of a sector, as shown below.

- Find the area of the shape enclosed by the wire.

- Give your answer correct to 1 decimal place.

- First work out θ using the perimeter:P=6.5+6.5+6.5θ

- 20=13+6.5θ

- θ=1413 rad

- Area=12×6.52×1413=22.75 cm2

Several sectors are to be cut from a single sheet of aluminium.

- By considering just the areas of the rectangle and sector, find the maximum number of pieces that can be cut from the plate.

- Area rectangle=2.5×1.2=3 m2

- Area sector=12×0.32×7π4=0.247400…m2

- Number of pieces=3÷0.247400…=12.1….

- So 12 pieces.

- Explain why practically this optimum number may not be possible.

- The sectors need to be whole pieces and as the diameter of the sector is 0.6 m. This means that you can fit 4 pieces across the length of the rectangle and 2 across the width which gives only 8 whole pieces that can be cut.

A system component has an area of 14.1 cm2 and a perimeter of 16.7 cm2. Given that the angle in the sector is acute, find the radius and angle of the sector.

- Draw a diagram

Area=12r2θ

Perimeter=2r+rθ

r

θ

rθ

r

- Use the information from the question to set up a pair of simultaneous equations.

- 12×r2θ=14.1          [1]

- 2r+rθ=16.7          [2]

- Rearrange [1] to make θ the subject and then substitute into [2] to eliminate θ.

- 1  θ=28.2r2

- 2 2r+r28.2r2=16.7

- 2r+28.2r=16.7

- 2r2+28.2=16.7r

- 2r2-16.7r+28.2=0

- r=6 or r=2.35

- when r=6 , θ=28.262=0.783 rad

- when r=2.35 , θ=28.22.352=5.11 rad (not acute)

- Final answer r=6 and θ=0.783 rad

The shape below is to be cut out of a plastic sheet.

- Find the perimeter of the shape.

100°=5π9 rad

Curved length=2×5π9=3.4906… m

Unknown length in triangle=3 m (Pythagorean triple)

Perimeter=5+4+3.4906…-3+2+3.4906…=15.0 m