4. Essential mathematics for engineering and manufacturing

# Worksheet 23: Averages and standard deviation (learner)

- A set of 20 ingots has been cast and their masses (in kg) are shown in the table below. Find the mean, median and standard deviation of the sample.

- A sample of 20 screws are taken and their length measured (in mm) from a batch being manufactured by a new pointer. Find the mean, median, mode and standard deviation of the sample.

| 18.2 | 18.0 | 18.0 | 18.1 | 18.3 | 17.8 | 17.9 | 17.9 | 18.0 | 18.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18.3 | 18.0 | 18.1 | 18.1 | 18.0 | 18.2 | 17.8 | 18.0 | 18.1 | 18.2 |

- A batch of resistors is tested with a ohmmeter and the resistance (in Ω) of each resistor is recorded below. Find the mean, median and standard deviation of the sample.

| 465.2 | 468.8 | 472.6 | 470.3 | 471.5 | 477.1 | 446.2 | 470.1 | 480.1 | 477.6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 458.9 | 449.1 | 446.0 | 468.7 | 470.9 | 448.7 | 485.4 | 449.3 | 450.8 | 479.3 |

- The weights of 20 cast components are recorded to the nearest 0.1 N and recorded in the table below. Find the mean, median and standard deviation of the sample.

| 8.2 | 8.0 | 8.0 | 8.1 | 8.3 | 7.8 | 7.9 | 7.9 | 8.0 | 8.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.3 | 8.0 | 8.1 | 8.1 | 8.0 | 8.2 | 7.8 | 8.0 | 8.0 | 8.0 |