5. Essential science for engineering and manufacturing

# Worksheet 10: Hydrostatic pressure (tutor)

- Divers must be careful when working at depth in water and need to carefully control the dissolving of gases in their bloodstream.

Calculate the pressure created by a 30 m depth of water given that the density of water is 1000 kg m−3 and the acceleration due to gravity 9.8 N kg−1.

P = hρg

P = 30 × 1000 × 9.8 
   = 294 000 Pa
   = 2.94 × 105 Pa (294 kPa)

- The density of sea water is ~1025 kg m−3 and the maximum depth of the Atlantic Ocean is ~8500 m (8.5 km).

Calculate the water pressure at this depth.

P = hρg

P = 8500 × 1025 × 9.8 
   = 85 400 000 Pa 
   = 85.4 MPa (to 3 s.f.)

- The pressure at the bottom of a storage tank of depth 12.5 m containing a chemical solvent is 306 kPa.

Calculate density of the solvent. Assume that the tank is full.

P = hρg,

Rearranging the formula gives

ρ  = P ÷ hg

= 306 000 ÷ (12.5 × 9.8)

= 2500 kg m−3 (to 3 s.f.)