5. Essential science for engineering and manufacturing

# Worksheet 8: Forces (tutor)

- Give an explanation and example for each of the following types of motion.

  - Linear

Linear motion involves movement in a straight line, for example a train moving down a straight, horizontal track.

  - Rotational

Rotational motion occurs when something moves around an axis or pivot point, for example a bicycle wheel.

  - Oscillating

Oscillating motion involves a curved backwards and forwards movement that swings on an axis or pivot point, for example a swing or a clock pendulum.

  - Reciprocating

Reciprocating motion has a repeated up and down movement or back-and-forth movement, for example a piston or a pump.

- Define the term ‘force’.

A force is a physical effect that when exerted on an object can cause it to move, change its state of motion or deform it.

- Define the three key characteristics of a force.

Magnitude, direction, point of application.

- For each collection of listed forces, determine the vector sum or the net force.

| Set A 58 N, right 42 N, left 98 N, up 98 N, down | Set B 14 N, left 16 N, up 16 N, down | Set C 12 N, up 8 N, down |
| --- | --- | --- |

Set A: ∑F = 16 N, right

Set B: ∑F = 14 N, left

Set C: ∑F = 4 N, up

- Two vehicles strike a lamppost simultaneously from different directions with the following forces:

FVehicle1 = 1230 N at 53°

FVehicle2 = 1450 N at 107°

Determine the resultant force applied by the two vehicles to the lamppost. (The directions of the two forces are stated as anticlockwise angles of rotation)

| Force | Horizontal component |  | Vertical component |  |
| --- | --- | --- | --- | --- |
| 1230 N at 53o | 1230 cos 53o | 740.2 N | 1230 sin 53o | 982.3 N |
| 1450 N at 107o | 1450 cos 107o | -423.9 N | 1450 sin 107o | 1386.6 N |
| Resultant |  | 316.3 N |  | 2368.9 N |

Resultant force R = 316.32+2368.92 = 2390 N to 3 significant figures

θ = tan-12368.9316.3 = 82.4o

Resultant force is 2390 N at 82°.