5. Essential science for engineering and manufacturing

# Worksheet 9: Pressure, force and area (learner)

- A force of 150 N is being applied over an area measuring 0.5 m2. Calculate the pressure on the object ensuring you give the correct units

- Pressure of 150 N m−2 is experienced when a force of 2 kN is applied. Calculate the area over which the force is applied to obtain the pressure stated.

- A cylinder sands on the circular end. The cylinder exerts a force of 2 N onto the floor. The cylinder has a radius of 0.5 m. Calculate the pressure that the cylinder exerts on the floor.

- The force applied to a 0.4 m by 0.8 m brake pad produces a pressure of 500 N m−2. Calculate the force applied to the brake pad.

- If a weight of fluid of 200 N acts on a surface of 5 m2, calculate the pressure created.

- What force must be applied to a surface area of 0.0025 m2, to create a pressure 
of 200 000 Pa?