5. Essential science for engineering and manufacturing

# Multiple choice questions (learner)

Derived units are obtained from various combinations of

- Base units.

- International standards.

- Electrical quantities.

- Metric prefixes.

An unconstrained steel bar is cooled from a temperature of 450 °C to 30 °C. How will this affect the length of the bar?

- The length will reduce.

- The length will stay the same.

- The length will increase.

- The length will increase then reduce.

The specific latent heat required to melt steel is 4200 kJ kg−1. Calculate the quantity of latent heat required to melt 15 kg of steel at its melting point.

- 63 kJ.

- 280 kJ.

- 42 MJ.

- 63 MJ.

What are the units of pressure?

- N m−2.

- m s−2.

- N m.

- kg m−3.

Which of the following is a scalar quantity in engineering mechanics?

  - Displacement.

  - Speed.

  - Velocity.

  - Acceleration.

6.2 × 109 seconds is equivalent to:

- 6.2 teraseconds.

- 6.2 picoseconds.

- 6.2 nanoseconds.

- 6.2 gigaseconds.

The degree of closeness of the measured value of a certain quantity with its true value is known as:

- Accuracy.

- Precision.

- Tolerance.

- Sensitivity.

Which of the following is used to check the diameters of holes?

  - Ring gauge.

  - Slip gauge.

  - Plug gauge.

  - Standard screw pitch gauge.

Which two particles make up the nucleus of an atom?

  - Protons and electrons.

  - Protons and neutrons.

  - Molecules and compounds.

  - Neutrons and electrons.

Which of the following contains only one type of atom?

  - Compound.

  - Molecule.

  - Element.

  - Solution.

Which of the following defines specific heat capacity?

  - The maximum of amount of heat that can be put into a substance before it melts.

  - The maximum amount of thermal energy that can be stored in one unit of mass of a substance.

  - The thermal energy given out when one unit of mass of a substance solidifies.

  - The thermal energy required to raise the temperature of one unit of mass of a substance by one degree.

An open system is one in which:

  - Mass does not cross boundaries of the system, though energy may do so.

  - Neither mass nor energy cross the boundaries of the system.

  - Both energy and mass cross the boundaries of the system.

  - Mass crosses the boundary but not energy.

Which term means irregular fluctuations or mixing during the movement of a fluid?

- Laminar flow.

- Vortex.

- Separation point.

- Turbulence.

A pneumatic system contains 1.5 m3 of air at 300 K. Calculate the volume of the gas when the temperature of the air is increased to 330 K at constant pressure.

- 1.35 m3.

- 1.50 m3.

- 1.65 m3.

- 1.80 m3.

An engineer needs to measure the internal diameter of a drilled hole to an accuracy of 
± 0.1 mm. What would be the most suitable item of equipment to make this measurement?

- Micrometer.

- Engineering rule.

- Vernier callipers.

- Feeler gauge.

A cylinder contains 10 m3 of gas at a pressure of 40 bar. Calculate the volume of the gas when the pressure is increased to 50 bar at constant temperature.

- 7.5 m3.

- 8.0 m3.

- 8.8 m3.

- 12.5 m3.

Calculate the amount of thermal energy needed to raise the temperature of 5 kg of steel 
by 300 °C.

The specific heat capacity of the steel is 480 J kg−1 °C−1.

- 720 kJ.

- 390 kJ.

- 240 kJ.

- 144 kJ.

Which of these statements describes what is meant by ‘conduction’.

- The transfer of thermal energy through thermal emission.

- The transfer of thermal energy through direct contact.

- The transfer of thermal energy through the movement of a gas.

- The transfer of thermal energy through the emission of light.

The temperature of an unconstrained 10 m long steel beam increased by 50 °C. The coefficient of linear expansion of the steel is 11 × 10−6 °C−1. Calculate the change in length of the beam.

- 0.055 mm.

- 0.55 mm.

- 5.5 mm.

- 55 mm.

Calculate the rate of heat transfer through a piece of material when ΔTm = 40 °C, A = 2 m2 and the overall heat transfer coefficient is 50 kJ h−1 m−² °C−1.

- 4 MJ h−1.

- 3 MJ h−1.

- 2 MJ h−1.

- 1 MJ h−1.