6. Materials and their properties

# Worksheet 12: Corrosion and corrosion prevention (learner)Top of Form

- Bottom of Form

1. Corrosion is an example of:

- Oxidation

- Reduction

- Electrolysis

- Erosion

2. Which of the following factors does not govern the rusting of iron?

- Presence of air

- Presence of moisture

- Presence of electrolytes in water

- Presence of impurities of more electropositive metals in iron

3. In galvanic corrosion:

- More metal gets corroded

- Less noble metal gets corroded

- The metal having a higher standard reduction potential gets corroded

- The metal placed lower in the electrochemical series get corroded

4. The localised formation of holes in the surface of a metal is called:

- Waterline corrosion

- Pitting corrosion

- Concentration cell corrosion

- Wet corrosion

5. What is the cause of crevice corrosion?

- A difference in the ion concentration of the surrounding air

- A difference in the ion concentration between two areas of metal

- A reduction in the ion concentration in one metal in relation to another

- An increase in the gap between two metals in close proximity to one another

6. Which of the following is not a method of corrosion prevention?

- Metal alloy

- Temperature control

- Chemical inhibitor

- Protective coating