6. Materials and their properties

# Worksheet 17: Interpreting results (tutor)Top of Form

- The following data have been obtained from conducting a tensile test on a material specimen of diameter 10mm and original length 60mm.

| Load (kN) | 16 | 32 | 56 | 72 | 95 | 110 | 132 | 142 | 140 | 135 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extension (mm) | 0.2 | 0.4 | 0.7 | 0.9 | 1.5 | 2.5 | 5.0 | 8.5 | 10.0 | 12.0 |

Using graph paper or a suitable computer package, plot a load–extension graph and determine the following for the sample material: tensile strength, 0.1% proof stress, Young’s modulus for the specimen.

Learners’ load-extension graph curve should resemble the graph below.

- The tensile strength of the material

Tensile strength 			=	Maximum load/cross-sectional area

=	142kN/(π x 102)

=	142/78.55

=	1.808 kN/mm2

- 0.1% proof stress

0.1% of 60mm = 0.06mm

Proof load = 77kN

0.1% proof stress 		=	Proof load/CSA

=	77 / 78.55

=	0.98KN/mm2

- Young’s modulus for the specimen

Find a point within the elastic region within diagram (eg, 32kN, 0.4mm)

Diameter 		= 	10mm

Length 		=	60mm

Young’s modulus 	=	Stress/strain

Stress 			=	Load/CSA

=	32/78.55

=	0.407 kN/mm2

Strain			=	Extension/original length

=	0.4/60

=	0.0067

Young’s modulus 	=	0.407/0.0067

=	60.7 kN/mm2