6. Materials and their properties

# Worksheet 1: Physical properties (tutor)

Create a mind map identifying some or all of the physical properties of materials you have learned about so far, giving examples. Two are provided to get you started.

Thermal conductivity

Thermal conductivity is a measure of the quantity of heat that flows through a material

Thermal expansion

Thermal expansion refers to a fractional change in the size of a material in response to a change in temperature

Melting point

In general, ‘melting’ is a phase change of a substance from the solid to the liquid phase

Permittivity

The ability of a material to store electrical potential energy under the influence of an electric field

Weldability

Weldability is the ability of any material (usually metals) to weld with a similar material

Corrosion resistance

Corrosion resistance describes a material’s ability to prevent natural chemical or electrochemical attack by atmosphere, moisture or other agents

Physical properties

Density

Density, often expressed as pounds per cubic inch, or grams per cubic centimetre, describes the mass of the material per unit volume

Hardenability

Hardenability is the ability of a steel to partially or completely transform from austenite to some fraction of martensite at a given depth below the surface, when cooled under a given condition

Specific heat capacity

Specific heat capacity is the amount of heat energy required to raise the temperature of a substance per unit of mass