6. Materials and their properties

# Worksheet 2: Mechanical properties (tutor)

Create another mind map identifying the different mechanical properties you have learned about, with a brief explanation of each.

Toughness

Measured using the Charpy impact test, toughness represents a material’s ability to absorb impact without fracturing at a given temperature

Strength – fatigue

Fatigue can lead to fracture under repeated or fluctuating stresses (eg loading or unloading) that have a maximum value less than the tensile strength of the material

Hardness

Defined as a material’s ability to resist permanent indentation

Brittleness

A mechanical property of a material manifested by failure without undergoing any deformation on application of stress

Elasticity

A material’s tendency to return to its original size and shape when a distorting force is removed

Strength – yield

Yield strength describes the point after which the material under load will no longer return to its original position or shape

Strength – shear

Shear strength is a consideration in applications like bolts or beams where the direction as well as the magnitude of the stress is important

Strength – tensile

The amount of load a section of material can withstand before it breaks

Mechanical properties

Ductility

The ability of a material to deform plastically (ie stretch) without fracturing and retain the new shape when the load is removed

Malleability

A material’s ability to be formed without breaking