6. Materials and their properties

# Worksheet 3: Density calculations (tutor)

- A bottle is filled with 2kg of oil which has a density of 0.925g/cm3. Calculate the volume of the oil to the nearest cm3. Calculate the volume of the oil to the nearest cm3.

Firstly, we must ensure that all units are the same, ie change the kg into grams to ensure that the solution is correct.

2kg = 2,000g

Volume = mass ÷ density

= 2,000 ÷ 0.925

= 2,162cm3

- A concrete block weighing 2,460kg has a volume of 1.2m3. Calculate the density of the block.

Density = mass ÷ volume

In this question, the mass is 2,460kg and the volume is 1.21m3, so we need to substitute these values into the formula as follows:

Density	 = 2,460 ÷ 1.2

= 2,050kg/m3

- Metal A has a density of 5g/cm3 and metal B has a density of 3g/cm3; 1,200g of metal A and 600g of metal B are melted and mixed and then recast into a block.

a) Calculate the volume of the block

b) Calculate the density of the block

In order to calculate the overall volume of the block, we need to add the volume of metal A and the volume of metal B. We don’t have the volume of either metal, but we have been given their masses and densities, so we can calculate the volume of each metal accordingly. By rearranging the density formula, or using the density triangle, we can work out how to calculate the volume:

Density = mass ÷ volume

Volume = mass ÷ density

The volume of metal A can be calculated as follows:

1,200g ÷ 5g/cm3 = 240cm3

The volume of metal B can be calculated as follows:

600g ÷ 3g/cm3 = 200cm3

Assuming no reduction in volume, if metal A has a volume of 240cm3 and B has a volume of 200cm3, then their combined volume is:

Volume of C 	= 240cm3 + 200cm3

= 440cm3

As we know from the answer to question a, the newly-formed block has a volume of 440cm3. We know that the mass of metal A was 1,200g and the mass of metal B was 600g, so the mass of the block is:

1,200g + 600g = 1,800g

The density of the block can be calculated by dividing the mass by the volume as follows:

Density  = mass ÷ volume

= 1,800g ÷ 440cm3

= 4.09g/cm3