6. Materials and their properties

# Worksheet 6: Thermoplastics, thermosets, composites and alloys (tutor)

- Using diagrams, show the difference between a thermoplastic and a thermoset material.

Thermoplastics can melt under heat after curing while thermosets retain their form and stay solid under heat once cured.

Thermoset resins (liquids or partially polymerised powders) are cured through a catalytic chemical reaction that crosslinks polymers to form an irreversible chemical bond. Once the crosslinks are formed, the product cannot be reshaped or remoulded

Heat is applied to thermoplastic polymers (solid pellets) to soften them. No chemical bonding occurs. The curing process is reversible. Thermoplastics can be reshaped or remoulded without compromising physical properties.

- What is a composite material? Give an example and an application in an engineering environment .

A composite material is made from two or more separate materials that are bonded or fused together, such as carbon fibre reinforced plastic (CRP) which consists of either an epoxy or polyester resin reinforced with carbon fibres. Used in racing cars.

- What is an alloy? What are the benefits of alloys?

An alloy is a metal made by combining two or more metallic elements, to give greater strength or resistance to corrosion.

Alloys offer multiple benefits that pure metals do not have. By using an alloy, you get the advantage of a blend of metals that combines the materials’ properties. Depending on the components in the alloy the following benefits can be achieved:

- corrosion resistance

- better solderability

- higher durability

- increased strength–weight ratio

- thermal conductivity

- electrical conductivity.