7. Mechanical principles

# Worksheet 7: Moments and reactions (learner)

Basic mechanical principles include the following:

- the theory of moments

- reactions.

Reaction left

Force

Reaction right

In order to answer the following questions, you will need to know some basic principles.

- Forces down must equal reactions up.

- A force or system of forces may cause an object to turn. A moment is the turning effect of a force. Moments act about a point in a clockwise or anticlockwise direction. Sum of anticlockwise moments = sum of clockwise moments.

- Bending moment = force × perpendicular distance.

Force (F) is measured in newtons (N) or kilonewtons (kN)

Distance (d) is measured in metres (m)

- If the weight of a uniform horizontal beam is 200kN and the distance between the supports at each end is 6m, what is the value of the reaction at each support?

- Calculate the maximum bending moment at the centre of the beam.