7. Mechanical principles

# Worksheet 7: Moments and reactions (tutor)

Basic mechanical principles include the following:

- the theory of moments

- reactions.

Reaction left

Force

Reaction right

In order to answer the following questions, you will need to know some basic principles.

- Forces down must equal reactions up.

- A force or system of forces may cause an object to turn. A moment is the turning effect of a force. Moments act about a point in a clockwise or anticlockwise direction. Sum of anticlockwise moments = sum of clockwise moments.

- Bending moment = force × perpendicular distance.

Force (F) is measured in newtons (N) or kilonewtons (kN)

Distance (d) is measured in metres (m)

- If the weight of a uniform horizontal beam is 200kN and the distance between the supports at each end is 6m, what is the value of the reaction at each support?

Taking moments about the right-hand end:

Reaction left × 6m = 200kN × 3m

Rleft=200 × 36=100kN

As the beam is symmetrical, the right reaction will also be 100kN

- Calculate the maximum bending moment at the centre of the beam.

At centre, maximum bending moment = 200 × 3 – 100 × 3 = 300kNm