7. Mechanical principles

# Multiple choice questions (tutor)

- A simply supported beam supports a single load at a single point along its length. What is the formula to calculate the support reaction at R1?

- R1 = (F x L1) / (L1 + L2)

- R1 = (F x L1) / (L2)

- R1 = (F x L2) / (L1)

- R1 = (F x L2) / (L1 + L2)

- The energy possessed by a flywheel because of its rotary motion is an example of:

  - Braking force

  - Rotational kinetic energy

  - Gravity

  - Tractive effort

- A vehicle has a mass of 800kg and is travelling at 1m/s-1. It collides with a second vehicle, which is stationary. The two become fixed together as a single mass travelling in the same direction as the original vehicle was at 0.4m/s-1. Assuming that no energy was lost in the collision, calculate the mass of the second vehicle.

  - 320kg

  - 480kg

  - 1120kg

  - 1200kg

- In which of the following force systems do all forces pass through a single point?

- Concurrent

- Kinetic

- Non-concurrent

- Coplanar

- Calculate the potential energy of a pallet with a mass of 70kg that is sitting on top of a platform 20m high (hint: gravity = 9.8m/s-2).

- 142.8J

- 882J

- 1400J

- 13 720J

- What is acceleration?

- Change in the velocity of an object over time, measured in m/s2

- Change in the velocity of an object over time, measured in m/s

- The motion of an object that is moving very fast

- Change in the position of an object over time, measured in m/s

- Newton’s first law states … ?

- Energy cannot be created or destroyed, just changed between different forms

- An object will remain at rest or in uniform motion in a straight line unless compelled to change its state by the action of an external force

- An object is in equilibrium if the forces acting on it are zero

- The acceleration of an object produced by a force is proportional to the mass of the object in the direction opposite to the force

- You measure a distance of 7m between two markers, and releasing a constant velocity object at one marker, you find it takes 4.2 seconds to travel between the markers. What is the speed of the object?

- 29m/s

- 1.67m/s

- 0.6m/s

- 7m/s

- Which of the following statements is true for a simply supported beam with uniformly distributed loading?

- The maximum value of the shear force is at the midpoint of the beam

- The bending moment diagram will be a sloping straight line passing through the midpoint

- The maximum value of the bending moment will be experienced at the two ends of the beam

- The reaction forces at each end of the beam will be the same

- A forklift truck with a mass of 800kg is travelling with a velocity of 40m/s-1. Calculate its kinetic energy.

- 32kJ

- 313kJ

- 640 kJ

- 1.28MJ

- Which of the following is an example of a contact force?

- Magnetic

- Electrostatic

- Friction

- Gravitational

- What is Newton’s second law in equation form?

- F = ma

- s=dt

- Force of friction = ma

- p = mv

- An object of mass 60kg is raised by a height of 15m. Calculate the change in the potential energy of the object (additional information: g = 9.81m/s-2).

- 588.6J

- 900J

- 2887J

- 8829J

- Which statement about frictional resistance is correct?

- It is a non-contact force

- It works in the same direction as the motion of the object

- It is the same irrespective of the materials used to make the surfaces

- Types include sliding, rolling and static friction

- A 50N force is applied to a shopping trolley, causing it to move a distance of 10m. How much work is done in this process?

- 500J

- 5J

- 0.2J

- 250J

- In which units is power measured?

- Joules

- Newtons

- Newton metres

- Watts

- A vehicle of mass 1500kg is initially travelling at 20m/s.  Calculate the braking force required for the car to come to a complete stop in 15m.

- 10kN

- 20kN

- 30kN

- 45kN

- In an experiment where a mass falls under gravity and turns a flywheel to which it is connected, what kind of energy exists in the system at the start?

- Kinetic energy

- Elastic potential energy

- Gravitational potential energy

- Rotational energy

- Which statement about tractive effort is correct?

- It reduces as the mass of the wheel increases

- It is not affected by gravity

- It increases as the friction coefficient between the wheel and surface increases

- It is a rotary force

- What is the formula for the kinetic energy of an object?

- KE = mgh

- KE = ½ mv2

- KE = mu

- KE = F/A