8. Electrical and electronic principles

# Worksheet 3: Test your knowledge (tutor)

The volt drop across a component is also the:

- Energy

- Charge

- Potential difference

- Power dissipated

A current of 3A is flowing through a resistance of 25Ω. What is the potential difference across the resistor?

- 50V

- 75V

- 100V

- 80V

What is the quantity of electricity when 5A of current flows for 3ms?

- 5m C

- 10m C

- 20m C

- 15m C

When measuring resistance in a circuit, which two things should you do?

- Isolate the component

- Keep the power supply on

- Connect the multimeter in series

- Turn off the power supply

What is the value of capacitance when 1C of charge is stored on a plate with 1V across it?

- 1uF

- 1pF

- 1F

- 1nF