8. Electrical and electronic principles

# Worksheet 4: Expand your knowledge (tutor)

Use your existing knowledge and conduct research as required to answer the following questions.

- Give reasons why you should never measure high currents with a multimeter. How can high currents be measured?

To measure currents with a multimeter, you have to connect it in series in a circuit. This means you would have to disconnect the circuit at some point and this could be dangerous. The current range of a multimeter is relatively low – usually a maximum of 10A. In circuits of greater power a clamp ammeter is used.

- Which instrument measures the insulation between a conductor and an earth?

An insulation resistance tester is used. This device puts a higher voltage across the insulation (typically twice the voltage or at least the working voltage). It measures the leakage current through the insulation, and expresses this as a resistance value.

- Give two examples of substances that can be used to make capacitors.

- Ceramic

- Electrolytic

- Film or paper

- A battery can supply a DC current. What is the difference between primary cell batteries and secondary cell batteries?

- Primary cell batteries are disposable when they are discharged

- Secondary cell batteries can be recharged

- How would you check a fuse with a multimeter?

- Remove the fuse from an isolated circuit.

- Select ohms on your mutimeter and connect the probes across the fuse.

- You should get zero ohms if the fuse is healthy, or infinity if the fuse has blown.