8. Electrical and electronic principles

# Worksheet 5: Series and parallel circuits (tutor)

-

Take a look at the worked example, them attempt questions 1 and 2.

Worked example:

- For the circuit shown, calculate:

- The total resistance 		49.6Ω

- The supply current		4.29A

- The supply voltage		213V

20Ω

10Ω         15Ω

10Ω

25Ω

V

6Ω            5Ω

20V

+

_

- Total resistance

First parallel branch of 6Ω and 5Ω

1Rt = 16 + 15    or   productsum   = 3011 = 2.73Ω

Using the same method for the 10Ω and 15Ω in parallel = 6Ω

As the two combinations are in series then that branch equates to 6 + 2.73 = 8.73Ω

The 8.73Ω is now in parallel with the 10Ω = 10×8.7310+8.73 = 4.66Ω

This results in three series connected resistors

20 + 4.66 + 25 = 49.66Ω

- Supply current

The 20V is across the combination 8.73Ω therefore current flowing I=208.73 = 2.29A

Current flowing through the 10Ω = 2010 = 2A

Total current is therefore 2 + 2.29 = 4.29A

- Supply voltage

V = (4.29 x 20) + (4.29 x 4.66) + (4.29 x 25) = 213V

- For the circuit shown, calculate

- The total resistance	13Ω

- The supply current	7.7A

15 Ω

10 Ω

_

100 V

+

20 Ω

15 Ω

10 Ω

10 Ω

5 Ω

- Total resistance

Combine the 10Ω and 5Ω and 10Ω together as they are in series = 10 +5 +10 = 25Ω

This combination is in parallel with the 10Ω so = 10×2510+25 = 7.14Ω

This resistance is in series with the 15Ω and second 15Ω = 15 +7.14 +15 = 37.14Ω

The 37.14Ω is then in parallel with the 20Ω = 20×37.1420+37.14 = 13Ω

- Supply current

I = V/R = 100/13 = 7.7A