8. Electrical and electronic principles

# Worksheet 6: Kirchhoff’s laws (tutor)

For the following circuit, calculate the current from each battery.

-15.9A and 18.14A

30v

0.2Ω

35v

0.1Ω

15Ω

I1

I2

I1 from the 30V battery

EQ2

EQ1

I2 from the 35V battery

Therefore I1 + I2 flowing through the 15Ω

resistor

Considering the left-hand loop and deriving the equation

In a clockwise route:

EQ1 30 = 15(I1 + I2) + 0.2I1, therefore 30 = 15.2I1 + 15I2	EQ1

Using the same method for EQ2 deriving in an anticlockwise direction:

EQ2 35 = 15(I1 + I2) + 0.1I2, therefore 35 = 15I1 + 15.1I2	EQ2

Solve the simultaneous equation:

I1 = -15.9A, I2 = 18.14A

For the circuit shown, which two

I2

of the following are true (hint: work out the currents first)?

20v

0.2Ω

18v

0.1Ω

10Ω

I1

- The 20V battery is being charged

- The 20V battery is being discharged

- The 18V battery is being charged

- The 18V battery is being discharged

EQ1 20 = 10(I1+I2) + 0.2I1, giving 20 = 10.2I1 +10I2

EQ2 8 = 10(I1+I2) + 0.1I2, giving 18 = 10I1 + 10.1I2

Solving the equations:

I1 = 7.28A and I2 = -5.43A

This means the 20V battery is discharging and the 18V battery is charging (b and c).