8. Electrical and electronic principles

# Worksheet 7: DC circuits (tutor)

- A capacitor of 500uF in series with a resistor of 200Ω is connected to a supply of 24V DC.

Calculate:

- The circuit time constant

- The maximum current

- The time taken for the capacitor to become fully charged

- The time taken for the capacitor to charge to 12V

- Ꞇ = C x R = 500uF x 200Ω = 100ms

- I = V/R = 24/200 = 120mA

- Time taken = 5Ꞇ = 5 x 100ms = 500ms

vCt=V(1-e-t/CR)

12 =24(1-e-t/100mS) transposing t = 69.3ms

- A capacitor is connected in series to a resistor of 2kΩ with a supply voltage of 150V DC. When the capacitor is fully charged the energy stored is 4.5J. Calculate the value of the capacitor.

W = 0.5 C V2

4.5 = 0.5 x c x 1502

= 400uF

- A resistance of 10Ω and inductance L are connected in series. The time constant of the circuit is 5ms. It is connected across a supply of 20V.

Calculate:

- The value of inductance L

- The circuit steady state current

- The value of current after 0.52ms

- Ꞇ = L/R

5mS = L/10Ω therefore L = 50mH

- I = V/R = 20/10 = 2A

it=2(1-e-0.52mS/5mS), therefore  i(t) = 0.198A

- A coil has a resistance of 25Ω and the final energy stored is 1.2J. The supply voltage is 50V DC.

Calculate:

- The coil inductance

- The circuit time constant

- The time taken for the maximum steady state current

- The time taken to reach 50% of the final steady state current

- W = 0.5 LI2, now I = 50/25 = 2A

1.2 = 0.5 x L x 22, therefore L= 0.6H

- Ꞇ = L/R = 0.6/25 = 24ms

- Time taken for maximum steady state current = 5Ꞇ = 5 x 24ms = 120ms

1 =2(1-e-t/24mS), therefore t = 16.6ms