8. Electrical and electronic principles

# Multiple choice questions (tutor)

The magnetic saturation of a material is approximately 1.5T. Is the material:

- Iron

- Copper

- Steel

- Aluminium

Electricity is concerned with the movement of:

- Neutrons

- Molecules

- Atoms

- Electrons

Fleming’s left-hand rule is best described as:

- First finger current, second finger field, thumb, force – describing generators

- First finger field, second finger current, thumb force – describing motors

- First finger field, second finger current, thumb force – describing generators

- First finger force, second finger current, thumb field – describing motors

What is best describes as a joules per second?

- Amps

- Coulombs

- Volts

- Watts

A power of 500W is dissipated in a resistor of 35Ω. What is the current and voltage?

- 3.78A and 132.29V

- 3.78A and 134.12V

- 3.76A and 132.29V

- 3.78A and 132 .58V

The energy stored in an inductor is 250mJ and the current is 5A. What is the inductance of the coil?

  - 35mH

  - 30mH

  - 25mH

  - 20mH

Consider the following illustration. What is the value of I4?

10A

25A

25A

I4

  - 40A

  - 35A

  - 25A

  - 35A

Consider the following circuit. What is the value of R1?

R total = 10.74Ω

10Ω

R1

25Ω

10Ω

15Ω

10Ω

5Ω

  - 20Ω

  - 10Ω

  - 15Ω

  - 25Ω

Consider the following circuit. Calculate the current through the 10Ω resistor.

35V

1Ω

37V

2Ω

10Ω

4.65A

  - 3.34A

  - 2.97A

  - 3.87A

A 100uF capacitor is connected in series with a resistor of 25Ω across a DC supply voltage. What is the time constant of the circuit?

  - 1.5ms

  - 2.5ms

  - 2ms

  - 1ms

An inductor of 150mH is connected in series with a 50Ω resistor across a 200V supply. Calculate the current after 4ms.

  - 3.52A

  - 3.62A

  - 2.95A

  - 2.65A

A 150uF capacitor is connected in series with a 50Ω resistor across a supply voltage of 150V. Calculate the time it takes the value of current to drop to 2A.

  - 4.35ms

  - 3.51ms

  - 2.54ms

  - 3.04ms

In an AC series circuit the value of inductive reactance (XL) is 25Ω and the value of capacitive reactance (XC) is 30Ω. Is the circuit current:

  - Lagging

  - Leading

  - In phase

  - Not enough data to determine

In an AC series circuit R = 40Ω, C = 200uF, L = 0.15H. The components are connected across a 240V AC supply at 50Hz. Calculate the value of the current.

  - 4.73A

  - 5.42A

  - 3.25A

  - 6.85A

If the current phasor is rotating behind the voltage phasor by 30 degrees, what is the power factor of the circuit?

  - 0.577 lag

  - 0.5 lead

  - 0.866 lead

  - 0.866 lag

When a diode is forward-biased, the potential difference across it is typically:

  - 1.0V

  - 0V

  - 0.7V

  - 0.1V

A npn transistor requires the following conditions to turn on:

  - The base -ve compared to the emitter

  - The base +ve compared to the collector

  - The base -ve compared to the collector

  - The base +ve compared to the emitter

Why must water purity be monitored on a water cooled transistor?

  - To make sure it has a low resistance

  - To avoid minerals building up in the cooling system

  - To make sure it has a high resistance

  - To avoid corrosion

If you were protecting a high voltage installation from lightning strikes, what would you use?

  - RCD

  - Overcurrent inverse time relay

  - Distance relay

  - Spark gap arrester

What devices would help connect a pressure transducer to a computer?

  - P/I connected to an A/D, connected to the computer

  - V/I connected to an A/D, connected to the computer

  - P/I connected to a D/A, connected to the computer

  - A/D connected to a A/D, connected to the computer