10. Engineering and manufacturing control systems

# Worksheet 2: Process blocks (tutor)

Complete the table by describing the function of each process block and give an example of how each might be used in a manufacturing control system.

| Process block | Function | Example of use |
| --- | --- | --- |
| Comparator | Compares a signal received with a reference signal to indicate which is bigger/smaller | Control system to ensure materials/stock are kept at a set (reference) temperature and to warn workers if this is exceeded |
| Counter | Adds up the number of signals or pulses received | Automatic counter on a production line to count how many products have been completed and are ready for packaging |
| Latch | Keeps a signal high or low until it is reset | Warning alarm for a product that has been manufactured out of tolerance, that stays on until the item has been checked |
| Pulse unit | Produces a continuous series of digital pulses | Creating pulses as stimuli to test the operation of electrical/electronic systems undergoing quality control checks |
| Timer | Keeps a signal high or low for a set period of time | Timing device for a manufacturing process to indicate how long the worker has to complete it before the product must move to the next stage of manufacture |

Any relevant alternative uses/applications can be accepted.

Explain how process blocks are shown in block systems diagrams.

The process function is represented as a square/rectangular block, with the signals going in and out of it represented by arrows. The type of process is labelled within the block (eg timer, counter etc). Process blocks are placed after input blocks but before output blocks.