11. Quality management

# Worksheet 6: Sampling and SPC (tutor)

Working in a small group, create a mind map that shows the differences between 100% sampling and SPC.

Feed back your findings to the other groups.

Most or all of the points below should be covered. Tutors may choose to give learners prompts or hints from column 1 (eg cost, time, fatigue, staff, etc) to encourage them to provide a thorough comparison.

|  | 100% sampling | SPC |
| --- | --- | --- |
| Cost | Expensive to operate | Cheaper to operate |
| Time | More inspection time required | Can be automated at very high speed |
| Fatigue | Inspectors may suffer from fatigue | Inspector fatigue mainly eliminated |
| Staff | More staff required | Less staff required as can be automated |
| Damage | More damage to products due to more handling | Less damage to products due to minimal handling |
| Destructive testing | Cannot be carried out | Can be carried out |
| Product lot | Lot may be saved as each piece is sampled | Entire lot may be rejected if a sample is found defective |