16. Continuous improvement

# Worksheet 2: KPIs (tutor)

- Define what is measured by the following KPIs.

- Throughput

Number of products manufactured per unit time.

- Lead time

Time to manufacture a customer order from start to finish.

- Changeover time

Time to switch a process between making different products.

- A process with a cycle time of 45 seconds produces 52 parts in an hour. Calculate the capacity utilisation.

Capacity utilisation = (output achieved / potential output) x 100

Capacity utilisation = ( 52 / (60 x 60 / 45)) x 100 = 65%

- A machining process produces 120 parts. On inspection, 9 were rejected as scrap.

- Calculate the percentage of products that were scrap.

Scrap = (total number of products scrapped / total products produced) x 100

Scrap = (9 / 120) x 100 = 7.5%

- Calculate the yield of the process.

Yield = 100 – scrap = 100 – 7.5 = 92.5%

Alternatively, yield = (good products produced / total products produced) x 100

= ((120 – 9) / 120) x 100 = 92.5%