16. Continuous improvement

# Multiple choice questions (learner)

What is meant by the waste of motion?

- The time spent by materials between processes

- The unnecessary movement of materials

- Moving parts twice between processes

- Any unnecessary movement by people

Making some extra parts in a batch in case of quality problems is an example of which type of waste?

- Overprocessing

- Overproduction

- Defects

- Contingency planning

3.	Which waste means not making full use of the skills and knowledge of workers?

- Unused talent

- Motion

- Over production

- Waiting

4.	What is meant by the waste of overprocessing?

- Making materials wait between manufacturing processes

- Making products that do not meet the expectations of customers

- Producing work that is higher quality than needed

- Making more products than the customer needs

5.	Which KPI means the number of products manufactured per unit time?

- Capacity utilisation

- Throughput

- Cycle time

- OEE

6.	What does OEE stand for?

- Overall environment effectiveness

- Overall environment efficiency

- Operating equipment efficiency

- Operating equipment effectiveness

7.	Which KPI means the time to switch a process between different products?

- Changeover time

- Lead time

- Waiting time

- Cycle time

8.	The output of a process in a one-hour period was 18 products. The cycle time for an individual product on the process was three minutes. Calculate the capacity utilisation.

- 54%

- 82%

- 90%

- 94%

9.	What does the D stand for in the PDCA cycle?

- Design

- Direct

- Do

- Develop

10.	Which visual management tool would be most effective to reduce the time spent searching for missing tools?

- Shadow board

- Progress board

- SOP

- Sight glass

11.	Which of the following is a purpose of a countdown board?

- To warn about hazards in the workplace

- To specify a method for carrying out an activity

- To communicate the number of parts made

- To show the number of products still to be made

12.	Which of the following is a limitation of visual management approaches?

- The information is not easily understood by workers

- Effort is required to keep the information up to date

- It reduces worker engagement

- It reduces focus on the presented information

13.	Which of the following is the correct listing of the 6Ss?

- Sort, set in order, shine, standardise, sustain, safety

- Sort, select, shine, systematise, sustain, safety

- Sort, straighten, shine, standardise, sustain, safety

- Sort, set in order, spruce up, schedule, sustain, safety

14.	Which step in the 6S process involves removing items that are not needed from the workplace?

- Set in order

- Schedule

- Sort

- Shine

15.	What is the purpose of the ‘sustain’ step when carrying out a 6S activity?

- To ensure that the process is environmentally friendly

- To maintain the benefits of the first four Ss

- To ensure that all resources used are renewable

- To establish standard procedures

16.	What does TPM stand for?

- Tracking production measures

- Total productive management

- Total preventative monitoring

- Total productive maintenance

17.	Which of the following is a typical benefit of carrying out a SMED activity?

- A reduction in the amount of tooling or jigs and fixtures required

- An increase in the operating equipment effectiveness of the process

- A reduction in the changeover time between products

- An increase in the size of the manufactured batches

18.	During a SMED activity, which of the following would have to be an internal element of the set-up process?

- Fetching raw materials

- Locating the tools needed for the set up

- Finding the correct paperwork

- Removal of guarding from equipment

19.	What is the purpose of kanban?

- To prevent the use of metal containers in the production area

- To manage the flow of work in a company

- To ensure that all processes are working at their maximum efficiency

- To reduce the time taken to change over between products

20.	Which of the following typically increases by implementing kanban?

- The amount of time needed for scheduling work

- The area needed to store work in progress

- The amount of inventory

- The number of defects produced